SSH Traffic Priority QoS Configuration on ens3 Server: s273032 ¦ Interface: ens3 ¦ Kernel: 6.8.0-111-generic

# Overview

SSH is how we remotely connect to and control this server. If the server gets busy, network traffic can get congested and SSH can become slow or drop out completely. Losing SSH access at the moment something is going wrong means losing the ability to fix it.

This document covers the complete configuration applied to give SSH traffic absolute highest priority over all other network traffic on interface ens3, for both IPv4 and IPv6.

Note: IPv6 u32 tc filters are not supported on kernel 6.8.0-111-generic. The IPv6 solution uses ip6tables packet marking combined with a tc fw filter instead.

# How It Works

Linux uses a tool called tc (Traffic Control) to manage how packets are queued and sent on a network interface. By default all traffic is treated equally. We change that by creating three priority bands on ens3:

- Band 1:1 — Highest priority (SSH traffic goes here)

- Band 1:2 — Normal priority (most traffic lands here by default)

- Band 1:3 — Lowest priority

The kernel works through band 1:1 completely before touching 1:2 or 1:3. So even under heavy load your SSH session keeps flowing.

For IPv4 we use tc u32 filters that match TCP port 22 directly. For IPv6 we use ip6tables to mark port 22 packets, then a tc fw filter that catches those marks and routes them into band 1:1.

# Step 1: Apply Rules Immediately

Run this block to apply all rules right now. It clears any existing rules first so it is safe to run multiple times.

Apply all rules (run as root)

# Remove any existing qdisc cleanly sudo tc qdisc del dev ens3 root 2>/dev/null ¦¦ true # Remove any existing ip6tables marks cleanly sudo ip6tables -t mangle -D OUTPUT -p tcp --dport 22 -j MARK --set-mark 1 2>/dev/null ¦¦ true sudo ip6tables -t mangle -D OUTPUT -p tcp --sport 22 -j MARK --set-mark 1 2>/dev/null ¦¦ true # Create the priority qdisc with default priomap sudo tc qdisc replace dev ens3 root handle 1: prio # IPv4: outbound SSH (destination port 22) to highest priority band sudo tc filter add dev ens3 protocol ip parent 1:0 prio 1 u32 \ match ip protocol 6 0xff \ match tcp dst 22 0xffff \ flowid 1:1 # IPv4: inbound SSH responses (source port 22) to highest priority band sudo tc filter add dev ens3 protocol ip parent 1:0 prio 1 u32 \ match ip protocol 6 0xff \ match tcp src 22 0xffff \ flowid 1:1 # IPv6: mark SSH packets via ip6tables sudo ip6tables -t mangle -A OUTPUT -p tcp --dport 22 -j MARK --set-mark 1 sudo ip6tables -t mangle -A OUTPUT -p tcp --sport 22 -j MARK --set-mark 1 # IPv6: route marked packets to highest priority band sudo tc filter add dev ens3 parent 1:0 prio 2 handle 1 fw flowid 1:1

# Step 2: Make It Survive Reboots

The tc rules and ip6tables marks are lost on reboot. This systemd service reapplies everything automatically every time the server starts, before other services come up.

Create and enable the systemd service

sudo tee /etc/systemd/system/ssh-qos.service > /dev/null <<EOF [Unit] Description=SSH QoS Priority After=network-online.target Wants=network-online.target [Service] Type=oneshot RemainAfterExit=yes ExecStart=/bin/bash -c '\ tc qdisc del dev ens3 root 2>/dev/null ¦¦ true; \ ip6tables -t mangle -D OUTPUT -p tcp --dport 22 -j MARK --set-mark 1 2>/dev/null ¦¦ true; \ ip6tables -t mangle -D OUTPUT -p tcp --sport 22 -j MARK --set-mark 1 2>/dev/null ¦¦ true; \ tc qdisc replace dev ens3 root handle 1: prio && \ tc filter add dev ens3 protocol ip parent 1:0 prio 1 u32 match ip protocol 6 0xff match tcp dst 22 0xffff flowid 1:1 && \ tc filter add dev ens3 protocol ip parent 1:0 prio 1 u32 match ip protocol 6 0xff match tcp src 22 0xffff flowid 1:1 && \ ip6tables -t mangle -A OUTPUT -p tcp --dport 22 -j MARK --set-mark 1 && \ ip6tables -t mangle -A OUTPUT -p tcp --sport 22 -j MARK --set-mark 1 && \ tc filter add dev ens3 parent 1:0 prio 2 handle 1 fw flowid 1:1' ExecStop=/bin/bash -c '\ tc qdisc del dev ens3 root; \ ip6tables -t mangle -D OUTPUT -p tcp --dport 22 -j MARK --set-mark 1; \ ip6tables -t mangle -D OUTPUT -p tcp --sport 22 -j MARK --set-mark 1' [Install] WantedBy=multi-user.target EOF sudo systemctl daemon-reload sudo systemctl enable --now ssh-qos.service

# Step 3: Verify Everything Is Working

Run these commands to confirm all rules are active.

Check tc rules

sudo tc qdisc show dev ens3 sudo tc filter show dev ens3

You should see output similar to this:

# qdisc show: qdisc prio 1: root refcnt 2 bands 3 priomap 1 2 2 2 1 2 0 0 1 1 1 1 1 1 1 1 # filter show: filter parent 1: protocol ip pref 1 u32 chain 0 filter parent 1: protocol ip pref 1 u32 chain 0 fh 800::800 ... *flowid 1:1 match 00060000/00ff0000 at 8 match 00000016/0000ffff at nexthdr+0 filter parent 1: protocol ip pref 1 u32 chain 0 fh 800::801 ... *flowid 1:1 match 00060000/00ff0000 at 8 match 00160000/ffff0000 at nexthdr+0 filter parent 1: protocol all pref 2 fw chain 0 handle 0x1 classid 1:1

Check ip6tables marks

sudo ip6tables -t mangle -L OUTPUT -v --line-numbers

You should see lines 2 and 3 with MARK set 0x1 for tcp dpt:ssh and tcp spt:ssh.

Check service status

sudo systemctl status ssh-qos.service

Expected output shows: active (exited) status=0/SUCCESS

# Removing Everything

To completely remove SSH QoS priority and stop it coming back on reboot:

sudo systemctl disable --now ssh-qos.service sudo tc qdisc del dev ens3 root sudo ip6tables -t mangle -D OUTPUT -p tcp --dport 22 -j MARK --set-mark 1 sudo ip6tables -t mangle -D OUTPUT -p tcp --sport 22 -j MARK --set-mark 1

# Technical Notes

## Why the default priomap is correct

The prio qdisc default priomap is: 1 2 2 2 1 2 0 0 1 1 1 1 1 1 1 1

This maps most normal traffic to bands 1:2 and 1:3. Our filters then explicitly push SSH into 1:1 (highest). Setting all priomap values to 0 (a common mistake) would put everything in 1:1, giving SSH no actual advantage.

## Why match tcp dst and not match ip dport

The tc u32 tcp selector uses dst and src keywords for ports, not dport and sport. Those belong to the ip selector. Using the wrong keyword causes tc to silently reject the filter.

## Why ip6tables for IPv6

On kernel 6.8.0-111-generic, tc u32 IPv6 filters with the ip6 selector fail with a kernel error. The ip6tables mark approach is the reliable workaround: ip6tables marks matching packets with a fwmark value of 1, and the tc fw filter at prio 2 catches anything with that mark and routes it to band 1:1.

## Why network-online.target

This server runs on QEMU/KVM (MAC prefix 52:54:00). On virtual machines, network.target does not guarantee that specific interfaces like ens3 are fully configured yet. network-online.target waits for at least one interface to be online before the service runs, making it reliable on reboot.

## Why the service cleans up before applying

Running tc filter add when filters already exist returns a 'Filter already exists' error and the service fails. Cleaning up first with 2>/dev/null || true means the service is fully idempotent. It can be started, stopped, and restarted any number of times without errors.